From: BITNET list server at PUCC <LISTSERV@pucc.PRINCETON.EDU> (1.7f)Subject: File: "HDINFO DOCS"To: Jon Bird <JRB@delphi.com>Message-id: <01HQDVXTQX2Y8ZPR76@delphi.com>Content-transfer-encoding: 7BIT     What's In This Document:     This document contains enough information for one to build the hardware     required to connect a PC/XT type hard disk controller to a coco. It also     includes enough information to write one's own drivers to access the hard     drive(s). If you want everything spelled out for you, this isn't going to     be what you want; this is for people who like to do it themselves.     Remember, this information is absolutely free-- buy a finished product from     someone if you want it all packaged and assembled for you.     Of course, the standard hardware project disclaimer stands as always. My     interface hasn't zapped anything that I've hooked it up to it. If you don't     feel comfortable mucking around with self-built hardware on your equipment     then DON'T!             Erik E. Petrich  11/19/89     Acknowledgement of trademarks:     BYTE is a registered trademark of McGraw-Hill Inc. IBM is a registered     trademark of International Business Machines Corporation. PC/XT is a     trademark of International Business Machines Corporation. Multi-Pak     Interface is a trademark of Tandy Corporation.     MBI might be a trademark of Noitaroproc Senihcam Ssenisub Lanoitanretni,     but only if it's an inconvience to you.     Thanks to:     James R. Drummond for a very handy article "Three Bus Interface Designs for     the PC" in Volume 12, Number 12 of BYTE.     Charles R. West for nagging me about my unfinished coco projects enough to     get this done.      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 1     Schematic:     Coco bus                                                  IBM PC/XT bus     D0(10)  ------------------------------------------------  D0(A9)     D1(11)  ------------------------------------------------  D1(A8)     D2(12)  ------------------------------------------------  D2(A7)     D3(13)  ------------------------------------------------  D3(A6)     D4(14)  ------------------------------------------------  D4(A5)     D5(15)  ------------------------------------------------  D5(A4)     D6(16)  ------------------------------------------------  D6(A3)     D7(17)  ------------------------------------------------  D7(A2)     5v(9)   ----------------------------------------*-------  5v(B3)                                                     |                                                     *-------  5v(B3)                                                     |     Gnd(33) ----*-----------------------------------|--*----  Gnd(B1)                 |                                   |  |     Gnd(34) ----*                                   |  *----  Gnd(B10)                                                     |  |                                                     |  *----  Gnd(B31)                                                     |  |                                                     |  |                                                     |  |     A0(19)  ----------------------------------------|--|----  A0(A31)     A1(20)  ----------------------------------------|--|----  A1(A30)     A2(21)  ----------------------------------------|--|----  A2(A29)     A3(22)  ----------------------------------------|--|----  A3(A28)                                                     |  |                                                     |  *----  A4(A27)                                                     |  |     A4(23)  ----------------------------------------|--|----  A5(A26)                                                     |  |                                                     |  *----  A6(A25)                                                     |  |                                                     |  *----  A7(A24)                                                     |  |                                                     *--|----  A8(A23)                                                     |  |                                                     *--|----  A9(A22)     ___                                                |     SCS(36)  ----------NOT--\                          |     Q(7)     ----NAND--NOT---NAND--*                   |     E(6)     ---/                  |                   *----  AEN(A11)                                    |                          ___                                    *------NAND--------------  IOR(B14)                           *--------|-----/                           |        |                          ___       _                   |        *------NAND--------------  IOW(B13)     R/W(18) --------------*--NOT---------/     _____     RESET(5)-----------------NOT----------------------------  RESET DRV(B2)      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 2     The connector on the coco side is a 40 land card edge. If a card is plugged     into the side of the coco, all the odd numbered pins are on top, all the     even numbered pins are bottom, with pin 1 closest to the back of the coco.     If a card is plugged into a Multi-Pak Interface, all the odd numbered pins     are on the side facing the back of the computer, all the even numbered pins     are on the side facing the front of the computer, with pin 1 farthest from     the coco.      ___________________________________________________________     |                                                           |     |                                                           |     |      This is the top view of a coco compatible card       |     |     drac elbitapmoc ococ a fo weiv mottob eht si sihT     |     |                                                           |     |_________                                         _________|               |          1 1 1 1 1 2 2 2 2 2 3 3 3 3 3|               |1 3 5 7 9 1 3 5 7 9 1 3 5 7 9 1 3 5 7 9|     Flipped over, it looks like this:      ___________________________________________________________     |                                                           |     |                                                           |     |      drad elbitapmoc ococ a fo weiv pot eht si sihT       |     |    This is the bottom view of a coco compatible card      |     |                                                           |     |_________                                         _________|               |4 3 3 3 3 3 2 2 2 2 2 1 1 1 1 1        |               |0 8 6 4 2 0 8 6 4 2 0 8 6 4 2 0 8 6 4 2|     The connector on the IBM end of things is a 62 land card edge. Each side is     numbered independently. The top side of the card is side "A". The bottom     side of the card is side "B". Pin 1 for each side is the land farthest from     the mounting bracket.      _______________________________________________________________________     |                                                                     |B|     |                This is the top view of an IBM type card             |r|     |               drac epyt MBI na fo weiv mottob eht si sihT           |k|     |                                                                     |t|     |                                                                     | |     |_____                   A A A A A A A A A A A A A A A A A A A A A A _|_|           |A A A A A A A A A 1 1 1 1 1 1 1 1 1 1 2 2 2 2 2 2 2 2 2 2 3 3|           |1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1|     Flipped over, it looks like this:      _______________________________________________________________________     |B|                                                                     |     |r|             darc epyt MBI na fo weiv pot eht si sihT                |     |k|           This is the bottom view of an IBM type card               |     |t|                                                                     |     | |                                                                     |     |_|_ B B B B B B B B B B B B B B B B B B B B B B                   _____|         |3 3 2 2 2 2 2 2 2 2 2 2 1 1 1 1 1 1 1 1 1 1 B B B B B B B B B|         |1 0 9 8 7 6 5 4 3 2 1 0 9 8 7 6 5 4 3 2 1 0 9 8 7 6 5 4 3 2 1|      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 3     Memory map:             Read            Write           Controller     $FF50   Data            Data            1     $FF51   Status          Reset           1     $FF52   Jumpers         Command Start   1     $FF53   undefined       Mode            1     $FF54   Data            Data            2     $FF55   Status          Reset           2     $FF56   Jumpers         Command Start   2     $FF57   undefined       Mode            2     $FF58   Data            Data            3     $FF59   Status          Reset           3     $FF5A   Jumpers         Command Start   3     $FF5B   undefined       Mode            3     $FF5C   Data            Data            4     $FF5D   Status          Reset           4     $FF5E   Jumpers         Command Start   4     $FF5F   undefined       Mode            4     Controllers come factory default as controller #1, but can usually be     rejumpered to #2, #3, or #4.      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 4     Initialization:     1. Set any value to the Reset register.     2. Write 0 to the Mode register.     Using the Controller:     1. Write any value to the Command Start register     2. Poll the Status register until the controller is

ready for a command        packet byte.     3. Send the next command packet byte to the Data register.     4. Loop to step 2 until entire command packet is sent.     5. Skip to step 9 if this command involves no data packet transfers.     6. Poll the Status register until the controller is ready to read/write        the next byte of the data packet.     7. Read/write the next byte of the data packet to/from the Data register.     8. Loop to step 6 until the entire data packet has been sent/received.     9. Poll the Status register until the controller is ready to write the        command completion code.     10. Get the command completion code from the Data register.     11. If bit 1 of the command completion code is set, an error occured.     Command packets are 6 bytes long and are in the following general format:             7 6 5 4 3 2 1 0     byte 1  ----command----     byte 2  0 0 d 0 h h h h         d=drive number, h=head     byte 3  c c s s s s s s         c=high bits of cylinder, s=sector     byte 4  c c c c c c c c         c=low bits of cylinder     byte 5  q q q q q q q q         q=interleave or sectors to transfer     byte 6  e e 0 0 r r r r         e=error correction strategy,                                     r=step rate.     NOTE: Not all fields are used for all commands.     e7 e6     0  0  No error correction attempted     0  1  No error correction attempted     1  0  Retry operation first, then attempt ECC correction     1  1  Attempt ECC correction immediately on error     r3 r2 r1 r0     0  0  0  0  3 msec step rate     all others  Vary somewhat from controller to controller. Consult your                 controller documentation for specific information.      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 5     Controller Commands:     Command  Function                       Data packet     -------  --------                       -----------     $00      Test drive ready               none     $01      Recalibrate drive              none     $03      Request sense status           4 bytes received     $04      Format drive                   none     $05      Check track format             none     $06      Format track                   none     $07      Format track bad               none     $08      Read sector                    512 bytes received     $09      Verify sector                  none     $0A      Write sector                   512 bytes sent     $0B      Seek track                     none     $0C      Initialize drive geometry      8 bytes sent     $0E      Format alternate track         3 bytes sent     $0F      Write sector buffer            512 bytes sent     $10      Read sector buffer             512 bytes received     Status register bits:     Bit 0   1=controller expecting to transfer data     Bit 1   0=controller expecting write,             1=controller expecting read     Bit 2   0=controller expecting data,             1=controller expecting command     Bit 3   0=command complete     If bits 0 and 3 are 0, then the controller is idling and ready to accept     another command.     If bits 0, 2, and 3 are 1, and bit 1 is 0, then the controller is expecting     the next byte of a command packet.     If bits 0 and 3 are 1, and bits 1 and 2 are 0, then the controller is     expecting the next byte of a data packet.     If bits 0, 1, and 3 are 1, and bit 2 is 0, then the controller has the next     byte of a data packet ready to be read.     If bits 0, 1, and 2 are 1, then the controller has the command completion     code ready to be read.      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 6     Command $00 - Test drive ready:     This command is generally used to make sure that the drive is turned on and     has spun up to operating speed. This command is also used to test if any     pending seeks have been completed.     Command $01 - Recalibrate drive:     This command seeks the heads to cylinder 0. Because this command     recalibrates the head positioning logic, it is considerably slower than a     direct seek command to cylinder 0.     Command $03 - Request sense status:     This command returns a 4 byte data packet detailing the status of a drive.             7 6 5 4 3 2 1 0     byte 1  l 0 t t e e e e         t=error type, e=error code,                                     l=valid location information (bytes 2-4)     byte 2  0 0 d 0 h h h h         d=drive number, h=head     byte 3  c c s s s s s s         c=high bits of cylinder, s=sector     byte 4  c c c c c c c c         c=low bits of cylinder     Command $04 - Format drive:     This command formats the drive starting at the location specified by bytes     2 through 4 in the command packet. Byte 5 of the command packet specifies     the sector interleave. If Bit 5 of byte 6 of the command packet is set, the     controller uses the current sector buffer contents for the default sector     data. Otherwise the controller uses a default value (usually $E6) for the     default sector data.     Command $05 - Check track format:     This command verifies that a track was properly formatted. Byte 5 of the     command packet specifies the sector interleave. Note that this command only     checks the format information-- the sector data is not verified as error     free.     Command $06 - Format track:     This command works identically to "Command $04 - Format drive" except that     it formats only 1 track.     Command $07 - Format track bad:     This command works identically to "Command $06 - Format track" except that     the track is marked as defective. All further sector read/write attempts to     this track will result in error type 1, code 9 "Track is flagged bad".      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 7     Command $08 - Read sector:     This command reads 1 or more sectors from the drive and returns a 512 byte     data packet for each sector successfully read. Byte 5 of the command packet     specifies how many sectors should be read.     Command $09 - Verify sector:     The command works indentically to "Command $08 - Read sector" except that     no data packets are returned.     Command $0A - Write sector:     This command writes 1 or more sectors to the drive and takes a 512 byte     data packet for each sector to be written. Byte 5 of the command packet     specifies how many sectors are to be written.     Command $0B - Seek track:     This command initiates a seek to the specified cylinder. Use "Command $03 -     Rquest sense status" to determine when the seek has completed. This command     is never required; all commands automatically perform seeks if required.     However, use of this command can improve disk performance in a     multi-processing environment when there are two disk connected to the     controller.     Command $0C - Initialize drive geometry:     This command takes an 8 byte command packet that describes the drive being     controlled. If the controller has 2 drives of different types connected,     this command needs to be issued whenever the controller switches between     them.             7 6 5 4 3 2 1 0     byte 1  c c c c c c c c  MSB total cylinders     byte 2  c c c c c c c c  LSB total cylinders     byte 3  0 0 0 0 h h h h  heads     byte 4  w w w w w w w w  MSB reduced write cylinder start     byte 5  w w w w w w w w  LSB reducded write cylinder start     byte 6  p p p p p p p p  MSB write precompensation cylinder start     byte 7  p p p p p p p p  LSB write precompensation cylinder start     byte 8  0 0 0 0 e e e e  Maximum ECC burst length correctable (max 11)      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 8     Command $0E - Format alternate track:     This command works identically to "Command $06 - Format track" except that     a 3 byte data packet is also taken specifying an alternate track. Any     accesses to the original track will automatically be re-routed to the     specified alternate track. This is an alternate strategy for dealing with     media defects from "Command $07 - Format track bad". The advantage to this     strategy is that the media can be made to appear defect free. The     disadvantage to this strategy is reduced disk performance due to additional     seek overhead.             7 6 5 4 3 2 1 0     byte 1  0 0 d 0 h h h h         d=drive number, h=head     byte 2  c c 0 0 0 0 0 0         c=high bits of cylinder     byte 3  c c c c c c c c         c=low bits of cylinder     Command $0F - Write sector buffer:     This command works identically to "Command $0A - Write sector" except that     no transfer is actually performed.     Command $10 - Read sector buffer:     This command works identically to "Command $08 - Read sector" except that     no transfer is actually performed. This command is used either as a quick     way to re-read the last sector accessed, or in conjunction with "Command     $0F - Write sector buffer" to verify the integrity of the controller's     memory.      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 9     Type 0 Error Codes:     $0 - No error occurred     $1 - No index signal from drive     $2 - No seek complete signal from drive     $3 - Write fault signal asserted by drive     $4 - Drive not ready     $6 - Track 0 not found     $8 - Drive is seeking (not necessarily an error)     Type 1 Error Codes:     $0 - ID field read error     $1 - Uncorrectable data error     $2 - No sector address marks found     $4 - Sector not found     $5 - Seek error     $8 - Correctable data

error encountered     $9 - Track if flagged bad     $A - Format error     $C - Direct access to an alternate track     $D - Alternate track already assigned     $E - Alternate track not found     $F - Alternate and defective track are the same     Type 2 Error Codes:     $0 - Invalid command     $1 - Illegal head, cylinder, or sector specified     $2 - Invalid parameter     Type 3 Error Codes:     $0 - Controller RAM failed     $1 - Controller firmware failed     $2 - Controller ECC generator failed      Note on Building/Using a Coco to PC/XT Hard Disk Interface  - Page 10

From:	IN%"LISTSERV@pucc.PRINCETON.EDU"  "BITNET list server at PUCC" 11-MAY-1995 17:48:28.26